A MINI PROJECT REPORT

On

SMART MOVE CHESS BOARD

RATHOD AVINASH

H.T. No. 2451-23-735-322

G.NITISH

H.T. No. 2451-23-735-320

B.E. (ECE) – V Sem. – ‘C’ Sec

Department of Electronics and Communication Engineering

(

)

MVSR ENGINEERING COLLEGE (A)

Sponsored by Matrusri Education Society, Estd. 1980

Affiliated to Osmania University & Recognised by AICTE

Nadergul (P.O.), Hyderabad - 501 510. R.R.Dt. T.S.

www.mvsrec.edu.in

2025- 2026

A MINI PROJECT REPORT

ON

SMART MOVE CHESS BOARD

Submitted to the

Dept. of Electronics and Communication Engineering,

M.V.S.R. Engineering College (A),

Osmania University, in partial fulfillment of

academic requirements for the award of the Degree

BACHELOR OF ENGINEERING IN

ELECTRONICS AND COMMUNICATION ENGINEERING

By

RATHOD AVINASH

H.T. No. 2451-23-735-322

G.NITISH

H.T. No. 2451-23-735-320

B.E. (ECE) – V Sem. – ‘C’ Sec

Department of Electronics and Communication Engineering

MVSR ENGINEERING COLLEGE (A)

)

(

Sponsored by Matrusri Education Society, Estd. 1980

Affiliated to Osmania University & Recognised by AICTE

Nadergul (P.O.), Hyderabad - 501 510. R.R.Dt. T.S.

www.mvsrec.edu.in

2025-2026

)

MVSR ENGINEERING COLLEGE (A)

(

Sponsored by Matrusri Education Society, Estd. 1980

Affiliated to Osmania University & Recognised by AICTE

Nadergul (P.O.), Hyderabad - 501 510. R.R.Dt. T.S.

www.mvsrec.edu.in

CERTIFICATE

This is to certify that the Mini Project Report that is submitted by

RATHOD AVINASH  2451-23-735-322

G.NITISH  2451-23-735-320

Studying   B.E  (ECE)  –  V Sem ‘C’  Sec  of  BACHLEOR  OF  ENGINEERING  in

ELECTRONICS  AND  COMMUNICATION  ENGINEERING   by   OSMANIA

UNIVERSITY, Hyderabad is a Bonafide work carried out by them during 5th Semester

Mr. G.Sravan Kumar                                        	                   Flg.Offr. DVSR Sesidhar

Prof., ECED                                             	                           	          Asst. Prof., ECED

Dr. K.Usha                                                                                     Dr.T.Srinivasulu

Asst. Prof., ECED                                                                                          Head, ECED

AHOD

# TABLE OF CONTENTS

# LIST OF FIGURES .............................................................................. vi
DECLARATION .................................................................................. vii
ACKNOWLEDGEMENT .....................................................................viii
ABSTRACT ............................................................................................ 9

# CHAPTER 1 : INTRODUCTION

# 1.1 Background......................................................................................
1.2 Problem Statement
1.3 Objectives of the Project
1.4 Proposed Methodology
1.5 Scope of the Project
Chapter Summary

# CHAPTER 2 : LITERATURE REVIEW

# 2.1 Existing Smart Chessboards
2.2 Input Detection Techniques (Reed, Hall, Optical)
2.3 LED-Based Indication Systems
2.4 Limitations in Existing Solutions
Chapter Summary

# CHAPTER 3 : SYSTEM OVERVIEW

# 3.1 System Architecture
3.2 Block Diagram
3.3 Overall Working Principle
Chapter Summary

# CHAPTER 4 : HARDWARE COMPONENTS

# 4.1 Reed Switches
4.2 Permanent Magnets
4.3 WS2812B Individually Addressable LED Strip
4.4 Arduino Mega 2560
4.5 Resistors, Power Supply & Protection Components
4.6 Chessboard Fabrication Materials
Chapter Summary

# CHAPTER 5 : SOFTWARE DESIGN

# 5.1 Matrix Scanning Algorithm
5.2 Row & Column Selection Logic
5.3 WS2812B LED Control (FastLED/NeoPixel)
5.4 Serpentine vs Non-Serpentine LED Mapping
5.5 Flowchart & Pseudocode
5.6 Arduino Source Code Overview
Chapter Summary

# CHAPTER 6 : IMPLEMENTATION

# 6.1 Board Fabrication Process
6.2 Placement of Reed Switches
6.3 Placement of Magnets in Chess Pieces
6.4 LEDs Arrangement Under Board
6.5 Wiring, Soldering & Assembly
6.6 Integration With Arduino Mega
6.7 Testing Procedures
Chapter Summary

# CHAPTER 7 : RESULTS AND DISCUSSION

# 7.1 Piece Detection Accuracy
7.2 LED Response and Visibility
7.3 Power Consumption and Stability
7.4 Observations
7.5 Limitations of the Current System
Chapter Summary

# CHAPTER 8 : APPLICATIONS

# 8.1 Educational and Learning Applications
8.2 Game Recording and Replay
8.3 Interactive Coaching Systems
8.4 Digital Chess Tournaments
Chapter Summary

# CHAPTER 9 : FUTURE SCOPE

# 9.1 Mobile App / Web Integration
9.2 Wireless (Bluetooth/Wi-Fi) Connectivity
9.3 Automatic Piece Recognition with Hall Sensors
9.4 AI Move Suggestions
9.5 Auto-Moving Pieces (Using Motors)
9.6 Cloud Game Storage
Chapter Summary

# CHAPTER 10 : CONCLUSION

# REFERENCES

# APPENDIX

# A. Arduino Code

# B. Circuit Diagrams
C. Additional Images

LIST OF FIGURES

Figure 1: Reed Switch .................................................................................. 1
Figure 2: Neodymium Magnets ................................................................... 2
Figure 3: WS2812B Addressable LEDs ....................................................... 3
Figure 4: Serpentine LED Wiring Layout ..................................................... 4
Figure 5: SMPS (Switched-Mode Power Supply) ........................................ 5
Figure 6: Arduino Mega 2560 ....................................................................... 6
Figure 7: Diodes Used in the Circuit .............................................................. 7
Figure 8: Implementation of Different Layers of the Chessboard ............. 8
Figure 9: Logic Testing and Verification ...................................................... 9

# DECLARATION

We, RATHOD AVINASH (2451-23-735-322) and G.NITISH (2451-23-735-320) students of MVSR ENGINEERING COLLEGE, Nadergul, studying III year V semester, ECE solemnly declare that the Mini Project Report, titled "SMART MOVE CHESS BOARD" is submitted to MVSR ENGINEERING COLLEGE for partial fulfillment for the award of degree of Bachelor of Technology in ELECTRONICS AND COMMUNICATION ENGINEERING.

It is declared to the best of our knowledge that the work reported does not form part of any      dissertation submitted to any other University or Institute for the award of any degree.

##### RATHOD AVIANSH

2451-23-735-322

##### G.NITISH

2451-23-735-320

# ACKNOWLEDGEMENT

We would like to express our gratitude to all the people behind the screen who helped us to transform an idea into a real application.

We express our sincere gratitude to our guide, Mr.G.SRAVAN KUMAR for providing continuous guidance, valuable insights, and unwavering support throughout the project.

We would like to thank our Project coordinator Flg.offr.DVSR SESIDHAR& our project Incharge Mr.G.SRAVAN KUMAR for their technical guidance, constant encouragement and support in carrying out our project at college.

We profoundly thank Dr. T. SRINIVASULU, Head of the Department of Electronics and Communication Engineering who has been an excellent guide and also a great source of inspiration to our work.

We would like to express our heart-felt gratitude to our parents without whom we would not have been privileged to achieve and fulfill our dreams. We are grateful to our principal, who most ably runs the institution and has had the major hand in enabling us to do our project.

The satisfaction and euphoria that accompany the successful completion of the task would be great but incomplete without the mention of the people who made it possible with their constant guidance and encouragement crowns all the efforts with success. In this context, we would like thank all the other staff members, both teaching and non-teaching, who have extended their timely help and eased our task.

##### RATHOD AVINASH

2451-23-735-322

##### G.NITISH

2451-23-735-320

# ABSTRACT

The Smart Move Chessboard is an interactive system developed to automatically detect the movement and placement of chess pieces while providing real-time visual feedback. Traditional chessboards require manual tracking of moves, which can be inconvenient for beginners, trainers, and digital game analysis. This project aims to modernize the chess-playing experience by integrating simple but effective sensing and lighting technologies without altering the natural feel of the game.

The board uses reed switches placed under each square and small magnets inside the pieces to identify their positions. Whenever a piece is placed or moved, the magnetic field activates the corresponding reed switch, allowing the system to know exactly which square is occupied. A matrix-scanning method implemented on the Arduino Mega enables efficient reading of all 64 squares with minimal wiring and fast response. The detected position data is then used to control WS2812B LEDs arranged beneath the board, which illuminate specific squares to indicate valid moves, highlight piece positions, or simply enhance the visual appeal of the game.

The project focuses on building a compact, low-cost, and reliable solution that avoids complex sensors while still achieving accurate detection. It demonstrates how basic electronic components can be combined with simple embedded programming to create a smart and interactive version of a familiar board game. The system can be used for chess training, move recording, demonstrations, or as a foundation for more advanced features such as mobile app connectivity or AI-assisted gameplay.

Overall, the Smart Move Chessboard showcases a practical application of sensors, microcontrollers, and LED control in a way that improves user experience while remaining easy to understand, assemble, and maintain

CHAPTER 1

INTRODUCTION

1.1 Background

Chess is one of the oldest and most intellectually engaging board games in human history. Over time, several digital versions of chess have emerged—mobile apps, PC games, AI-based opponents, and online multiplayer platforms—making the game more accessible and interactive. However, despite this digital evolution, the traditional physical chessboard continues to offer a unique tactile experience that digital interfaces cannot replicate. Many students, beginners, and enthusiasts still prefer learning and playing chess on a physical board because it provides better spatial understanding, piece handling, and comfort.

Yet, traditional chessboards lack two critical features that modern learners need:

- Guidance on legal moves, and

- Error prevention when a player makes an illegal or accidental move.

Beginners frequently struggle to remember how each piece moves, what checks or pins exist on the board, or whether a move is legal according to standard chess rules. This leads to frustration, slower learning, and reduced interest in the game. To address these issues, modern learning tools attempt to combine physical gameplay with digital assistance.

The Smart Move Chess Board is a system designed exactly for this need. It integrates sensors, LEDs, and microcontroller logic directly into a physical chessboard. This allows players—especially beginners—to receive real-time guidance directly on the board without needing a phone or computer screen.

1.2 Motivation for the Project

The idea for this project started from observing how beginners often struggle with move legality while learning chess. New players frequently ask questions like:

- “Where can this piece move?”

- “Is this move allowed?”

- “Why is this check?”

The existing solutions to this problem are mostly software-based chess engines or mobile apps. While these tools are excellent, they do not offer the physical interaction and hand–eye coordination that a real chessboard provides. There is a clear need for a hybrid system that preserves the traditional board experience while offering digital support.

Additionally, this project was motivated by the desire to create an embedded systems–based educational tool. It combines multiple technical domains:

- Sensor interfacing

- Embedded C programming

- LED control and power management

- Matrix scanning

- Real-time decision making

- Game logic implementation

This makes it an ideal mini-project for ECE students, as it applies theoretical concepts to a real-world system.

1.3 Problem Statement

Traditional chessboards do not provide any feedback regarding:

- Legality of moves

- Possible squares a piece can move to

- Whether the move exposes the king to danger

- Whether an illegal capture or illegal displacement occurred

Beginners often feel overwhelmed because remembering the rules of each piece requires time, practice, and repeated mistakes. Without guidance, learning becomes slow.

Therefore, a system is needed that:

- Detects the physical movement of each chess piece,

- Understands which piece is being moved,

- Shows only the legal squares for that piece, and

- Alerts the user when an illegal action occurs.

This forms the core problem that the Smart Move Chess Board aims to solve.

1.4 Objectives of the Project

The major objectives of this project are:

- To design a physical chessboard embedded with sensors that detect piece movement.

- To implement a real-time LED-based guidance system showing legal moves.

- To write a chess engine algorithm capable of calculating legal moves, checks, and captures.

- To prevent illegal moves by blocking or warning the user.

- To create a board that can be used as a teaching aid for beginners.

- To demonstrate the concepts of embedded systems, sensors, electronics, and microcontroller programming.

1.5 Scope of the Project

This project mainly focuses on:

- Detecting the presence or absence of chess pieces using reed switches.

- Mapping detected moves to the internal game state stored in Arduino Mega.

- Using WS2812B LEDs to display legal moves (green), captures (red), checks (yellow), and game events.

- Ensuring the movement follows standard chess rules for all major pieces.

However, some advanced rules like castling, en-passant, and automatic pawn promotion are currently not implemented. These can be added in future versions.

1.6 Methodology Overview

The Smart Move Chess Board follows a well-structured methodology:

- Piece Detection:
Each square contains a reed switch. When a magnet-equipped chess piece is placed on a square, the reed switch closes and sends a digital signal to the Arduino Mega.

- Matrix Scanning:
The board is wired in an 8×8 matrix. Arduino scans the rows and columns to identify which reed switches are active.

- State Machine Analysis:
The Arduino determines whether the player lifted a piece, placed it on a legal square, attempted an illegal movement, or tried to capture incorrectly.

- Move Generation:
Based on the type of piece, the board generates legal moves using predefined algorithms.

- LED Guidance:
WS2812B LEDs illuminate legal squares using colors, helping the beginner understand possible moves.

- Error Handling:
Illegal moves trigger red LEDs or an error animation until the player corrects the mistake.

1.7 Applications of Smart Move Chess Board

- Beginner-friendly chess learning aid

- Training tool for schools, chess academies, and hobby clubs

- Demonstration project for embedded systems courses

- Assistive tool for visually challenged learners (with sound integration in future)

- Digital–physical hybrid board for game-based learning

1.8 Significance of the Project

The Smart Move Chess Board bridges the gap between traditional gameplay and modern interactive learning. It enhances:

- Learning speed

- Understanding of chess rules

- Engagement and motivation

- Error-free gameplay

- Practical embedded systems skill development

This makes it not only a useful gaming tool but a meaningful educational innovation.

1.9 Chapter Summary

This chapter introduced the motivation, background, and need for a Smart Move Chess Board. It presented the objectives, scope, methodology, and applications of the system. The chapter establishes the foundation for understanding why such a system is valuable for both learning and embedded system innovation.

CHAPTER 2

LITERATURE REVIEW

The purpose of this chapter is to present a comprehensive overview of existing research, systems, and technologies relevant to the Smart Move Chess Board. A strong literature review not only establishes the background of this project but also highlights the gaps that justify why this mini-project is meaningful and necessary.

2.1 Overview of Smart Chess Systems

Smart chessboards are not entirely new, but they have evolved significantly over the years. Early designs in the 1980s used mechanical sensors and pressure switches to detect movements. These systems were expensive, slow, and limited in accuracy.
In the last decade, improvements in embedded systems, sensors, and low-cost microcontrollers have enabled more refined designs. Today’s commercial smart chessboards (e.g., Square Off, DGT boards) use:

- Hall Effect sensors

- Magnets in pieces

- Bluetooth / WiFi connectivity

- Mobile app integration

- AI chess engines

While these boards are advanced, they are also expensive and not beginner-focused. Most commercial boards target tournament players or online players, not students who need guidance and visualization.

The Smart Move Chess Board differs because it is designed as an educational tool, not a high-end online chess platform.

2.2 Reed Switch–Based Detection Systems

Reed switches are magnetic sensors used in many DIY smart chessboards. Several hobby projects exist online using:

- 64 reed switches

- 64 magnets

- 8×8 matrix scanning

- Arduino Mega or Uno

These projects mostly aim to detect piece positions. However, many limitations exist:

Limitations of existing reed-switch chessboards

- They only detect presence/absence, not move legality.

- They do not perform real-time LED guidance.

- Many are prototypes without stable wiring or stable power distribution.

- They lack a robust game engine (legal move logic).

- They do not handle incorrect lifts, captures, or illegal sequences.

Most reed-switch chessboards online simply light up when a square is occupied.
In contrast, your project adds logic, rule enforcement, visual feedback, and a complete embedded game engine.

2.3 Hall Effect Sensor–Based Smart Boards

Some advanced boards use Hall sensors (A3144, SS495A, etc.) which give analog or digital output based on magnetic field strength.
Advantages include:

- Higher durability

- More accurate sensing

- Can even differentiate piece types (if magnet strength varies)

But disadvantages are:

- Higher cost

- Requires per-square calibration

- More wiring complexity

- Requires multiple PCBs or multiplexers

For beginners and low-cost educational systems, reed switches remain the most practical choice.

Thus, the literature shows why your design is cost-effective and appropriate for student-level implementation.

2.4 RFID / NFC–Based Chessboards

A few research projects use RFID tags embedded in chess pieces and RFID readers under the board. These systems can:

- Identify exact piece type

- Track movement history

- Provide accurate detection regardless of height

However, they have serious limitations:

- High cost

- Needs custom PCBs

- Requires dozens of tag readers

- Slower reading if many tags overlap

- Hard to replicate as a student project

Therefore, RFID-based systems remain impractical for simple embedded designs.

2.5 Camera-Based Chess Recognition

Modern AI vision systems can detect chess piece movements using:

- Overhead cameras

- Image segmentation

- Object detection algorithms (YOLO, OpenCV, TensorFlow)

- Board state reconstruction

Although this method eliminates the need for physical sensors, it introduces:

- Complex software

- Requires constant lighting

- High processing power

- Costly hardware (Raspberry Pi + camera module)

For a physical, LED-guided beginner board, camera vision is unnecessary and overly complicated.

Your reed-switch approach is more reliable for real-time embedded processing.

2.6 LED-Guided Educational Tools

Certain educational tools use LEDs to guide beginners through puzzles or learning exercises:

- Piano learning mats that light up keys

- Mathematics boards with LEDs indicating correct options

- Circuit learning kits with LED paths

Applying this same principle to chess is relatively new.
Very few systems combine:

- Sensors

- LED guidance

- Game logic enforcement

This makes your project unique because it merges:

- Sensor-based learning tools

- Real-time embedded rule enforcement

- Chess algorithm logic

- Visual feedback loops

Such systems enhance learning by providing instant feedback, a known accelerator for skill development.

2.7 Arduino-Based Chess Projects

A large body of online resources showcase Arduino-based chessboards.
However, most fall under one of these categories:

Category 1: Basic reed-switch detection

- Only detects piece presence

- No LED display

- No move validation

Category 2: LED matrix boards (WS2812 or RGB strips)

- Only display patterns

- No chess logic

- No actual game play

Category 3: Mixed prototypes

- Use LEDs + sensors

- But lack complete rule implementation

2.8 Gap Analysis

After studying existing systems, the following gaps are evident:

Gap 1: Lack of beginner guidance

Most smart boards assume the player already knows chess.

Gap 2: Lack of real-time LED assistance

Few systems highlight legal moves directly on the board.

Gap 3: No handling of illegal movements

Existing prototypes simply accept any movement.

Gap 4: No complete embedded chess engine

Students rarely implement full rule logic in C on a microcontroller.

Gap 5: Cost vs. technology mismatch

Hall sensor– or RFID-based systems are expensive for academic projects.

Gap 6: No combination of physical and digital learning

Most projects focus either on physical detection or digital gameplay, not both.

2.9 Summary of Literature Review

This chapter reviewed several existing smart chess systems and related technologies such as reed switches, Hall sensors, RFID systems, and camera-based recognition. It was observed that although many systems exist, they tend to focus on either detection or display, but rarely on integrated rule enforcement with real-time LED guidance.
Your project fills this important gap by combining:

- A physical board

- Sensor-based detection

- LED-based move indication

- Embedded rule enforcement

Thus, the Smart Move Chess Board stands out as a cost-effective, educational, and technically robust system suitable for beginners and embedded systems learners alike.

CHAPTER 3

SYSTEM DESIGN

The Smart Move Chess Board is an embedded system that integrates hardware sensing, LED-based visual output, and microcontroller-based logic to detect player actions and enforce chess rules in real time. This chapter describes the detailed system architecture, design methodology, block diagrams, data flow, functional modules, and internal operation cycles that define the behavior of the board.

The design process followed a bottom-up approach: beginning with the basic sensing element (reed switches), then constructing the matrix, integrating the microcontroller, implementing LEDs, and finally creating a software engine capable of playing and validating the game.

3.1 System Architecture Overview

At its core, the Smart Move Chess Board consists of three major subsystems:

1. Input Subsystem – Piece Detection Matrix

- An 8×8 grid of reed switches

- Magnets embedded in chess pieces

- Diode-based matrix wiring

- Column and row scanning by Arduino Mega

This subsystem provides the board’s real-time input, indicating the current physical state of the pieces.

2. Processing Subsystem – Embedded Chess Engine

The Arduino Mega performs:

- Matrix scanning

- Move detection (lift/place events)

- Illegal move filtering

- Legal move generation

- Check and checkmate logic

- Next-turn logic

- Animation and LED output logic

The microcontroller executes a state machine that tracks the sequence of events during gameplay.

3. Output Subsystem – LED Feedback Layer

Beneath the chessboard is an 8×8 array of WS2812B LEDs:

- Shows legal moves (green)

- Shows capture moves (red)

- Shows checks (yellow)

- Shows errors (red blink)

- Shows start-up and win animations

This transforms the chessboard into a visually guided, interactive learning tool.

3.2 Block Diagram of the System

The block diagram represents the high-level interaction between subsystems:

Explanation:

- When a piece is placed or lifted, the magnetic field changes the reed switch state.

- The matrix sends these switch states to the Arduino.

- Arduino detects:

  - A lift event

  - A placement

  - A capture attempt

- Arduino runs the chess logic to determine:

  - Whether the move is legal

  - Whether a check or checkmate has occurred

- LED matrix lights up to guide the user visually.

3.3 Functional Decomposition

To simplify development, the system was divided into independent functional modules:

3.3.1 Input Processing Module

Responsible for converting physical sensor states into digital board coordinates.

Functions include:

- Matrix scanning

- Ghosting and bounce reduction

- Tracking piece lift and placement

- Synchronizing board state with movement events

3.3.2 Game Logic Module

The brain of the system, which includes:

- Identification of piece type

- Generating all legal moves for that piece

- Detecting errors

- Checking for check/checkmate

- Managing turn order

This module ensures that the game follows strict chess rules.

3.3.3 LED Control Module

Controls visual feedback:

- Illuminates available legal moves

- Indicates captures

- Displays startup animation

- Shows check/checkmate alerts

- Highlights invalid actions

- Shows power-on diagnostics

WS2812B LEDs use a serial protocol where Arduino sends a timed digital signal to control:

- Color

- Brightness

- Animation speed

3.3.4 Error Handling Module

Handles irregular situations:

- Illegal lifts

- Wrong turn

- Invalid placements

- Multiple lifts without placement

- Inconsistent board state

When errors occur, the board enters STATE_ERROR, and LEDs flash red until the user corrects the wrong action.

3.3.5 State Machine Controller

Every piece movement is analyzed using a state machine with states such as:

- Idle

- Holding

- Capture Pending

- Wrong Turn

- Error

- Game Over

This ensures deterministic handling of all player actions.

3.4 Hardware–Software Co-Design

The Smart Move Chess Board is an excellent example of co-design, where hardware and software are optimized together.

Hardware supports:

- Fast response times

- Minimal ghosting

- Noise-free detection

- Logical wiring layout

Software ensures:

- Correct interpretation of sensor data

- Stable game logic

- Smooth LED control

- Recovery from user mistakes

Both layers operate in harmony to form a seamless, responsive system.

3.5 Data Flow in the System

The data flow is structured and predictable:

Step 1 – Physical Action

A player lifts or places a chess piece.

Step 2 – Change in Magnetic Field

Reed switch opens or closes.

Step 3 – Matrix Scan

Arduino reads all 64 squares every 40 ms.

Step 4 – Event Detection

The software determines whether it was:

- A lift

- A placement

- A capture

- An illegal interaction

Step 5 – Logic Processing

Chess engine evaluates legality:

- Movement pattern

- Obstructions

- Check safety

- Turn order

Step 6 – Output Generation

LED matrix lights up with:

- Legal move squares

- Capture squares

- Error indicators

- Check alerts

3.6 Turn-Based Game State

A major requirement was ensuring that players follow the rules of alternating turns.

Current Turn Variable

A variable (currentTurn) tracks whether it is:

- Player White’s turn

- Player Black’s turn

This prevents beginners from:

- Moving multiple pieces in one turn

- Moving opponent’s pieces

- Manipulating the board incorrectly

LED Behavior Based on Turn

White’s pieces glow light blue during their turn.
Black’s pieces glow darker blue during their turn.

3.7 Color Logic Design

Color differentiation is crucial for clarity:

| LED Color | Meaning |
| --- | --- |
| Green | Legal move square |
| Red | Illegal move or wrong capture attempt |
| Yellow | King in check |
| Blue tones | Current player pieces |
| Teal/Cyan | Startup sequence |
| Purple/Custom | Win animation |

This provides intuitive visual learning for beginners.

3.8 System Operation Cycle Summary

The system follows a repeating cycle:

- Scan reed switch states

- Detect changes

- Update board logic

- Check move legality

- Generate LED instructions

- Render LEDs

- Return to scanning

This loop runs ~25 times per second, providing real-time responsiveness.

3.9 Chapter Summary

This chapter presented the complete system design of the Smart Move Chess Board, covering architectural details, block diagrams, functional decomposition, and real-time system workflow. Through hardware–software co-design and a well-structured logic flow, the system achieves reliable, intuitive, and interactive chess guidance for beginners. The next chapter will explore the components used in detail with theoretical background.

CHAPTER 4

HARDWARE COMPONENTS

This chapter presents an in-depth study of all major hardware components used in the Smart Move Chess Board. Each component was chosen carefully to ensure reliability, cost-effectiveness, and compatibility with the embedded system design. Beyond simply listing parts, this chapter explains how each component works, the theory behind its operation, and why it is suitable for this project. This theoretical understanding also prepares the reader for implementation and troubleshooting.

4.1 Reed Switches

4.1.1 Introduction

A reed switch is a small electromagnetic device consisting of two thin ferromagnetic metal blades enclosed within a hermetically sealed glass capsule. When subjected to a magnetic field, these blades are pulled together, completing an electrical circuit. Reed switches are widely used in sensing applications such as security alarms, door sensors, and—most relevant here—piece detection in chessboards.

In this project, 64 reed switches are placed under each chess square to detect whether a magnetic chess piece is present.

4.1.2 Theory of Operation

The reed switch operates purely based on magnetic induction. The two reeds inside the glass envelope are:

- Made of nickel–iron alloy

- Positioned with a slight gap

- Coated with a thin layer of rhodium for long-life contact

When magnet is near → blades attract → switch closes.

When magnet is removed → blades separate → switch opens.

This makes reed switches ideal for binary detection (ON/OFF), matching the requirements of the chessboard.

Fig 1 : REED SWITCH

4.1.3 Magnetic Sensitivity

Reed switches are categorized by their operating sensitivity, measured in A·T (Ampere-Turns) or Gauss.

The common sensitivity levels are:

- 10–20 A·T (low sensitivity)

- 20–40 A·T (medium sensitivity → used in this project)

- 40+ A·T (high sensitivity)

Low sensitivity requires a stronger magnet and is not ideal for small chess pieces.
High sensitivity leads to false triggering.

Thus, medium sensitivity was selected for reliable piece detection without ghosting.

4.1.4 Why Reed Switches?

There are three primary sensor choices for smart chessboards:

| Parameter | Reed Switch | Hall Sensor | RFID Tag |
| --- | --- | --- | --- |
| Cost | Very Low | Medium | High |
| Wiring | Simple | Medium | Complex |
| Durability | Medium | High | High |
| Power Use | None | Uses power | Uses power |
| Coding | Simple | Complex | Complex |
| Accuracy | High | Very High | Very High |

Reed switches are the best balance for a student-level embedded project.

4.1.5 Limitations

- Fragile due to glass body

- Must be soldered carefully

- Magnets must be aligned precisely

- Can break under excessive force or heat

These limitations influenced the assembly process significantly.

4.2 Neodymium Magnets

Magnets embedded in the chess pieces create the magnetic field needed to activate reed switches.

4.2.1 Type and Dimensions

The magnets used were:

- Neodymium (N35 grade)

- 5 mm diameter × 3 mm thickness

Neodymium magnets were chosen because:

- They are strong even at small sizes

- They provide consistent field intensity

- They activate reed switches reliably

Fig : 2  Neodymium Magnets

4.2.2 Magnetic Field Characteristics

The magnetic field of circular magnets is strongest at:

- The center

- The face of the magnet

Thus, magnets were fixed at the bottom-center of each chess piece.

Spacing considerations:

- Too strong → triggers nearby squares

- Too weak → fails to activate the reed

- Optimal distance → 4–8 mm above the reed switch

The board height was reduced to maintain this sensitivity threshold.

4.2.3 Placement Challenges

Several issues occurred during assembly:

- Magnets had polarity orientation issues

- Pieces had uneven bottoms

- Poor alignment caused inconsistent detection

Solutions included:

- Using hot glue for stability

- Ensuring equal magnet depth

- Testing each square individually

4.3 WS2812B Addressable RGB LEDs

WS2812B LEDs (NeoPixels) provide the visual output of the board.

4.3.1 LED Structure

Each WS2812B LED contains:

- Red, Green, Blue LEDs

- Integrated control IC

- Serial data interface

This makes them individually addressable with only one data line.

Fig 3 : WS2812B LEDs

4.3.2 Communication Protocol

WS2812B LEDs operate using an 800 kHz NRZ (Non-Return-to-Zero) protocol.

Timings:

| Signal | Time (ns) |
| --- | --- |
| T0H | ~350 ns |
| T0L | ~900 ns |
| T1H | ~900 ns |
| T1L | ~350 ns |

- Logic ‘0’ → short HIGH, long LOW

- Logic ‘1’ → long HIGH, short LOW

The Arduino Mega generates these precise timings.

4.3.3 Serpentine LED Wiring

The LED strip is arranged in a zig-zag serpentine pattern:

- Row 0 → Left to right

- Row 1 → Right to left

- Row 2 → Left to right

- … and so on

This requires a coordinate mapping function:

int ledIndex(int r, int c) {

if ((r & 1) == 0) return r*8 + c;

else return r*8 + (7 - c);

}

Without this mapping, LED output appears scrambled.

Fig 4 Serpentine LED Wiring

4.3.4 Power Requirements

Each LED can draw up to:

- 60 mA at full white brightness

Thus:

- 64 LEDs × 60 mA → 3.84 A maximum

A 5V 5A SMPS was selected to provide stable power.

To avoid startup spikes:

- Brightness was capped at 140

- Animations avoided full-white bursts

Fig  5 : Switched Mode Power Supply 5V 5A

4.4 Arduino Mega 2560

Arduino Mega is the “brain” of the system.

4.4.1 Key Features

- 54 digital I/O pins

- 16 analog inputs

- 256 KB Flash memory

- Robust timers and interrupts

This project required:

- 16 I/O for the matrix (8 rows + 8 columns)

- 1 pin for LED data

- Memory for large chess state arrays

- Heavy logic for move generation

Uno or Nano cannot handle this efficiently.

4.4.2 Port Usage in This Project

| Component | Pins Used |
| --- | --- |
| Reed Switch Rows | 39, 41, 43, 45, 47, 49, 51, 53 |
| Reed Switch Columns | 4, 5, 6, 7, 8, 9, 10, 11 |
| WS2812B LED Data | Pin 13 |
| Serial Debug | TX/RX |

Rows are outputs, columns are inputs with pull-up.

Fig 6 : Arduino Mega 2560

4.4.3 Why Arduino Mega?

- Supports the heavy chess logic

- Large RAM capacity

- Excellent library support

- Handles real-time scanning and LED rendering simultaneously

4.5 Matrix Wiring (Rows, Columns, and Diodes)

4.5.1 Purpose of Matrix Wiring

Wiring 64 reed switches individually would require 64 input pins—impossible on most microcontrollers.

Matrix wiring reduces this to:

- 8 rows

- 8 columns

- Total: 16 pins only

4.5.2 Ghosting Problem

When wiring grids without diodes, ghosting occurs where:

- A pressed switch falsely activates another square

- Caused by current flowing backward through wiring paths

Solution: Use 1N4148 diodes

Placed between every reed switch and row line to prevent reverse current.

Fig 7 :  Diodes

4.5.3 Matrix Scanning Algorithm

for(int r = 0; r < 8; r++) {

digitalWrite(rowPins[r], LOW);

delayMicroseconds(60);

for(int c = 0; c < 8; c++) {

curState[r][c] = (digitalRead(colPins[c]) == LOW);

}

digitalWrite(rowPins[r], HIGH);

}

This identifies exactly which reed switches are active.

4.6 Power Supply Design

A 5V 5A SMPS powers both the LEDs and logic.

4.6.1 Challenges Faced

- Wrong capacitor selection (47 µF instead of 1000 µF) led to:

  - Voltage drops

  - LEDs receiving only 2V

  - Flickering and random data corruption

- Common ground was missing initially → LEDs displayed garbage colors.

These issues were carefully resolved.

4.7 Physical Structure

- Base made from cardboard sheets

- Reed matrix mounted under cardboard

- LED panel under the reed grid

- Glass sheet placed on top for premium look

- Chessboard pattern placed between glass and LED grid

4.8 Chapter Summary

This chapter described in detail all hardware components used in the Smart Move Chess Board. Theories behind sensor operation, LED communication, microcontroller functioning, and matrix wiring principles were explored. These components form the foundational layer upon which the software and logic of the system operate. The next chapter focuses on the software architecture and algorithms.

CHAPTER 5

SOFTWARE DESIGN

The software running inside the Arduino Mega forms the “intelligent core” of the Smart Move Chess Board. While the hardware detects physical movement, it is the software that interprets those inputs, applies chess rules, prevents illegal moves, controls LED feedback, and ensures real-time responsiveness. This chapter presents a detailed explanation of the software architecture, including state machines, logic flows, data structures, legal move generation algorithms, LED rendering logic, and error handling mechanisms.

The program is written entirely in Embedded C++ using Arduino frameworks, optimized for speed, memory efficiency, and smooth user interaction.

5.1 Software Architecture Overview

The software design is divided into five major layers:

1. Input Layer

Handles reed switch scanning and state changes.

2. Game Logic Layer

Implements:

- Piece identification

- Move legality rules

- Capture logic

- Check and checkmate detection

3. State Machine Layer

Manages:

- Lift events

- Placement events

- Corrective actions

- Errors

- Game progress

4. LED Rendering Layer

Controls:

- Legal move indicators

- Capture indicators

- Error animations

- Startup and win animations

5. Utility Modules

Include:

- Coordinate translation

- Serpentine LED mapping

- Board resetting

- Debug logging

Together, these layers ensure coherent functioning of the board.

5.2 Data Structures Used in the Code

Data structures are carefully designed for memory efficiency.

5.2.1 Board Representation

int pieceBoard[8][8];

This matrix stores the current board state.

Values:

| Piece | Value |
| --- | --- |
| White Rook | 1 |
| White Knight | 2 |
| White Bishop | 3 |
| White Queen | 4 |
| White King | 5 |
| White Pawn | 6 |
| Black Rook | 9 |
| Black Knight | 10 |
| Black Bishop | 11 |
| Black Queen | 12 |
| Black King | 13 |
| Black Pawn | 14 |
| Empty | 0 |

This numerical representation enables fast processing.

5.2.2 Legal Move Grid

bool legal[8][8];

Filled with true for all squares where the lifted piece may legally move.

5.2.3 Reed Switch State Grids

bool curState[8][8];

bool lastState[8][8];

Used to detect transitions:

- last = true, cur = false → Lift event

- last = false, cur = true → Placement event

5.3 State Machine Design

A state machine ensures predictable behavior no matter what the player does.

5.3.1 The Defined States

enum GameState {

STATE_IDLE,

STATE_HOLDING,

STATE_CAPTURE_PENDING,

STATE_GAME_OVER,

STATE_ERROR,

STATE_WRONG_TURN

};

STATE_IDLE

No piece is being moved.
Board waits for a lift event.

STATE_HOLDING

Player has lifted their piece.
Legal moves are shown.

STATE_CAPTURE_PENDING

A capture is initiated by lifting an opponent's piece.
Player must place their piece on the captured piece's square.

STATE_WRONG_TURN

Triggered when a player lifts an opponent’s piece.

STATE_ERROR

Triggered when:

- A piece is lifted incorrectly

- Multiple squares change unexpectedly

- An illegal placement occurs

STATE_GAME_OVER

Checkmate condition reached.

5.3.2 State Transition Diagram

Below is a conceptual illustration:

Illegal moves or wrong turn cause transitions to ERROR or WRONG_TURN.

5.4 Matrix Scanning Routine

The input layer involves repeatedly scanning all 64 reed switches.

Code Snippet:

void scanMatrix(bool dest[8][8]) {

for(int r = 0; r < 8; r++) {

digitalWrite(rowPins[r], LOW);

delayMicroseconds(60);

for(int c = 0; c < 8; c++) {

dest[r][c] = (digitalRead(colPins[c]) == LOW);

}

digitalWrite(rowPins[r], HIGH);

}

}

Key design choices:

- Row pulled LOW activates that row for scanning

- Columns use INPUT_PULLUP

- 60 microsecond delay ensures settling time

- The loop runs every cycle (40 ms interval)

This ensures instant detection of physical movement.

5.5 Legal Move Generation Algorithms

This is the core of the chess engine inside the Arduino.

The project implements legality for:

- Pawn

- Rook

- Knight

- Bishop

- Queen

- King

Castling, en-passant, and automatic pawn promotion are excluded.

5.5.1 Helper Functions

Bounds Check

bool inBounds(int r, int c) {

return r >= 0 && r < 8 && c >= 0 && c < 8;

}

Color Identification

bool isWhitePiece(int p) { return p >= 1 && p <= 6; }

bool isBlackPiece(int p) { return p >= 9 && p <= 14; }

bool sameColor(int a, int b) {

return (isWhitePiece(a) && isWhitePiece(b)) ||

(isBlackPiece(a) && isBlackPiece(b));

}

5.5.2 Simulation for Checking King Safety

To ensure no move exposes the king to danger:

bool isMoveLegalSim(int r0, int c0, int r1, int c1, int type) {

int savedDest = pieceBoard[r1][c1];

int savedSrc  = pieceBoard[r0][c0];

pieceBoard[r1][c1] = type;

pieceBoard[r0][c0] = 0;

bool check = isCheck(isWhitePiece(type), pieceBoard);

pieceBoard[r0][c0] = savedSrc;

pieceBoard[r1][c1] = savedDest;

return !check;

}

This prevents illegal self-check moves.

5.5.3 Rook & Queen (Straight Moves)

void genSlide(int r0, int c0, int type, int dr, int dc) {

for(int d = 1; d < 8; d++) {

int r = r0 + dr*d;

int c = c0 + dc*d;

if(!inBounds(r,c)) break;

int target = pieceBoard[r][c];

if(target == 0) tryAddMove(r, c, type);

else {

if(!sameColor(type, target)) tryAddMove(r, c, type);

break;

}

}

}

5.5.4 Knight Moves

const int dr[8] = {-2,-2,-1,-1,1,1,2,2};

const int dc[8] = {-1,1,-2,2,-2,2,-1,1};

5.5.5 King Moves

All 8 adjacent squares are tested.

5.5.6 Pawn Logic

Handles double-step moves, forward moves, and diagonal captures.

This is one of the most complex portions due to directional movement and starting-row rules.

5.6 LED Rendering Engine

The LED engine converts board state into visual feedback.

5.6.1 Serpentine Coordinate Mapping

int ledIndex(int r, int c) {

if ((r & 1) == 0) return r * 8 + c;

return r * 8 + (7 - c);

}

5.6.2 LED Overlay Buffer

uint8_t ledBuffer[64];

This layer allows the system to combine:

- Piece background color

- Legal move color

- Captures

- Errors

- Animations

5.6.3 Rendering Logic

void renderLEDs() {

FastLED.clear();

for(int r=0; r<8; r++) {

for(int c=0; c<8; c++) {

int idx = ledIndex(r,c);

uint8_t lb = ledBuffer[idx];

if(pieceBoard[r][c] != 0) {

// base color for piece

if(isWhitePiece(pieceBoard[r][c]))

leds[idx] = CHSV(160, 10, 120);

else

leds[idx] = CHSV(150, 30, 80);

}

// apply overlays

switch(lb) {

case LB_MOVE: leds[idx] = CRGB::Green; break;

case LB_CAPTURE: leds[idx] = CRGB::Red; break;

case LB_CHECK: leds[idx] = CRGB::Yellow; break;

case LB_ERROR: leds[idx] = CRGB(255,40,40); break;

}

}

}

FastLED.show();

}

5.7 Error Handling and Recovery

Errors are a natural part of learning chess. Common triggers:

- Lifting the wrong piece

- Attempting to capture illegally

- Placing a piece on a forbidden square

- Changing multiple squares simultaneously

The system responds with:

- Red error LEDs

- Display freeze

- Forced correction (user must return the piece)

Once corrected, normal play resumes.

5.8 Startup, Win, and Diagnostic Animations

Animations include:

- Cyan wave at startup

- Teal ripple sequence

- King-highlight animation on victory

- Random wave pattern for checkmate

These enhance user experience and visual engagement.

5.9 Chapter Summary

This chapter presented a detailed explanation of the Smart Move Chess Board software system. It described data structures, state machines, legal move generation algorithms, matrix scanning logic, LED rendering, and error handling. Together, the software layers form a robust engine that transforms simple sensor inputs into a rich, rule-enforced, interactive chess-playing experience.

CHAPTER 6

CIRCUIT DIAGRAM & BLOCK DIAGRAM EXPLANATION

A strong embedded system project requires clear understanding of both the logical (block-level) and electrical (circuit-level) design. This chapter provides a full explanation of the circuitry used in the Smart Move Chess Board, detailing how reed switches, diodes, LEDs, and the microcontroller work together. The interaction between hardware blocks ensures stable sensing, accurate move detection, and responsive LED output.

This chapter expands on earlier system-overview concepts and focuses specifically on signal flow, electrical pathways, matrix wiring principles, diode protection, and LED driver requirements.

6.1 Block Diagram (System-Level Overview)

Although the physical system is complex internally, its top-level representation is straightforward. Below is the conceptual block layout of the Smart Move Chess Board.

6.2 Explanation of Block-Level Interaction

6.2.1 Magnet + Reed Switch Interaction (Input Trigger Block)

Each chess piece contains a 5×3 mm neodymium magnet at its base. When placed on a square, the magnet activates the reed switch beneath, allowing Arduino to detect:

- Piece presence

- Piece lift

- Piece placement

6.2.2 Reed Switch Matrix (Input Layer)

64 reed switches are arranged into an 8×8 matrix, reducing the wiring from 64 individual pins to 16 pins (8 rows + 8 columns). This matrix:

- Minimizes wiring complexity

- Enables multiplexed scanning

- Requires diode isolation to avoid ghosting

Reed switch closures indicate exactly which square is occupied.

6.2.3 Arduino Mega (Processing Layer)

Arduino Mega reads matrix states and processes:

- Game turns

- Legal moves

- Captures

- Checks

- Errors

- LED mapping

It acts as the brain of the system.

6.2.4 LED Matrix (Output Layer)

WS2812B LEDs visualize:

- Where the lifted piece can legally move

- Illegal attempts

- Captures

- Check and checkmate

- Start-up animations

Color-coded feedback instantly guides players, making learning intuitive.

6.3 Reed Switch Matrix Circuit

The 8×8 reed switch matrix is the core sensing component.

6.3.1 Basic Concept

Each reed switch is positioned at the intersection of:

- One row line

- One column line

When a magnet closes the switch, the row and column become electrically linked.

6.3.2 Wiring Strategy

Reed wiring rule:

- One terminal of each reed connects to its row line

- The other terminal passes through a 1N4148 diode into the column line

The diode prevents current backflow.

This makes the system scan-safe, meaning:

- No ghost readings

- No accidental triggers of adjacent squares

- Accurate sensing of multiple simultaneous lifts (if needed)

6.3.3 Why Diodes Are Necessary (Ghosting Problem)

If diodes were not used, consider this scenario:

- Two squares are pressed in the same row

- Their column lines might short together

- Arduino would misread additional squares as active

This is known as ghosting, common in matrix keyboards.

Adding a diode in series with each reed switch guarantees:

- Current flows only one way

- No false activations

- Multiple switches can be reliably scanned

6.3.4 Column Input Configuration

Columns are configured with:

pinMode(colPins[c], INPUT_PULLUP);

Meaning:

- Default state → HIGH

- When a reed is pressed → LOW

This allows simple, noise-free sensing.

6.4 Arduino Connection Diagram (Pin Mapping)

| Module | Pin Numbers | Configuration |
| --- | --- | --- |
| Reed Switch Rows | 39, 41, 43, 45, 47, 49, 51, 53 | OUTPUT |
| Reed Switch Columns | 4, 5, 6, 7, 8, 9, 10, 11 | INPUT_PULLUP |
| WS2812B LEDs | 13 | Digital Output |
| Power Input | Vin / 5V | External 5A SMPS |

Arduino pins were chosen to ensure:

- Distinct row/column spacing

- Low electrical noise

- No conflicts with timer-dependent functions

6.5 WS2812B LED Circuit

6.5.1 LED Connections

The LED matrix has three main terminals:

- 5V → From SMPS

- GND → Common ground

- DIN → Arduino pin 13

WS2812B LEDs are chained, meaning:

- Output of LED 1 → Input of LED 2

- Output of LED 2 → Input of LED 3

- … and so on

Only one pin is needed for control.

6.5.2 Importance of Common Ground

Without a shared ground between:

- Arduino Mega

- WS2812B LEDs

- SMPS

the data signal becomes unstable.

Symptoms of missing ground:

- Wrong colors

- Flickering

- Random animations

- LEDs not responding at all

You experienced this issue and fixed it by connecting all:

GND (Arduino) → GND (LEDs) → GND (SMPS)

6.5.3 Decoupling Capacitors

WS2812B datasheets recommend:

- 1000 µF capacitor across 5V and GND

- 300–500 Ω resistor on DIN line

In your project:

- A mistake using 47 µF caused voltage drop

- LEDs received only 2V

- Causing flicker and malfunction

Replacing with correct 1000 µF stabilized the system.

6.6 Power Supply and Electrical Load

WS2812B LEDs require significant current.

Worst-case calculation:

- 64 LEDs × 60 mA = 3.84 A

- Additional for Arduino ~ 150 mA

- Total required: ≥4 A

You selected a 5V 5A SMPS, providing safe overhead.

6.6.1 Voltage Drop Issues

Because LEDs are current-intensive:

- Using thin wires can cause voltage drop

- Long LED strips accumulate resistance

Solutions implemented:

- Thick power lines to LEDs

- Direct power injection to LED grid

6.7 Integration of Hardware Blocks

The system integrates multiple components into one interactive board.

Input → Processing → Output Loop

- Reed switch closes → Arduino detects LOW

- Arduino updates pieceBoard[][]

- Move legality is calculated

- LED instructions generated

- LEDs light accordingly

- System returns to scanning state

This loop repeats ~25 times per second.

6.8 Example Circuit Walkthrough

Let’s walk through an example:

A white knight is lifted from (r=0, c=1):

- Reed at (0,1) opens → row 0 HIGH, col 1 HIGH

- Arduino detects lastState != curState

- pieceBoard[0][1] changes to 0

- HeldPiece set to 2 (Knight)

- generateLegalMoves() identifies up to 8 squares

- LED matrix marks them green

- Player observes guidance

If placed incorrectly:

- System enters STATE_ERROR

- Square glows red

- Only returning the piece corrects the error

This interaction demonstrates how electrical and logical components work in combination.

6.9 Chapter Summary

This chapter provided a thorough explanation of the circuit-level and block-level design of the Smart Move Chess Board. It covered the reed switch matrix, diode isolation, Arduino pin mapping, LED driver circuitry, serpentine wiring, and power supply considerations. These components are essential to ensuring smooth system operation, accurate movement detection, and stable LED performance.

CHAPTER 7

IMPLEMENTATION

This chapter describes how the Smart Move Chess Board was physically built, step-by-step, from concept to final working prototype. While the earlier chapters focused on theory, design, and architecture, this chapter explains the real-world construction process, including mechanical assembly, wiring, soldering, testing, debugging, and integration of hardware with software. It also documents the difficulties faced and the solutions applied.

This chapter is intentionally detailed to reflect actual implementation experience, making the report realistic, academically strong, and technically complete.

7.1 Overview of Implementation Process

The project was implemented in the following stages:

- Planning and design finalization

- Creating the reed switch matrix

- Creating the LED matrix

- Arduino wiring and interface setup

- Integrating diodes for ghosting prevention

- Mounting components inside the board

- Testing individual components

- Integrating software and hardware

- Debugging and iterations

- Final enclosure and finishing

7.2 Preparing the Base Structure

The physical chessboard was designed with:

- Cardboard base for supporting wiring

- Insulated copper wires arranged as row and column buses

- 64 square grid marked with exact spacing based on a 30 × 30 cm board

- Glass top for professional appearance and uniform lighting

Fig 8 :Implementation of different layers

The board consists of layers:

Top Glass Sheet (for aesthetics)

↓

Printed Chessboard Layout

↓

LED Matrix Layer (8×8 WS2812B)

↓

Reed Switch Matrix Layer

↓

Base Cardboard Support

Each layer had to be aligned carefully to ensure magnets triggered only their designated reed switches.

7.3 Building the Reed Switch Matrix

7.3.1 Cutting and Preparing the Grid

- A cardboard sheet was measured and partitioned into 8 × 8 equal squares.

- Holes were created to position reed switches exactly at the center of each square.

- This ensured consistency of detection.

7.3.2 Laying Row and Column Wires

Two sets of insulated wires were arranged:

- 8 horizontal wires → rows

- 8 vertical wires → columns

These wires form the backbone of the matrix.

7.3.3 Mounting Reed Switches

Each reed switch was:

- Inserted into the square

- Bent slightly for stable contact

- Hot-glued carefully because reed switches are made of glass and fragile

You mentioned cracking multiple reeds initially due to pressure and heat, so later switches were handled more gently.

7.3.4 Adding Diodes (Ghosting Prevention)

Each reed switch’s one terminal was connected through a 1N4148 diode to the column wire.

Orientation:
→ Anode toward reed, cathode toward column wire

This prevented ghost readings and made scanning reliable.

7.3.5 Soldering Work

This was one of the most time-consuming and challenging parts:

- 64 reed switches × 64 diodes = 128 solder joints

- Additional wiring required for rows and columns

- Soldering errors initially caused:

  - Open circuits

  - Shorts

  - Damaged reeds

  - Burn marks on hand

  - Melting of insulation

Final wiring was neat and stable after multiple corrections.

7.4 Building the WS2812B LED Matrix

7.4.1 Choosing LED Arrangement

The LEDs were arranged in a serpentine pattern:

- Row 0 → left to right

- Row 1 → right to left

- Row 2 → left to right

- ... and so on

This was required because LED strips internally follow a continuous physical path.

7.4.2 Mounting LEDs

The LED strip or individual LEDs were:

- Positioned evenly under each square

- Fixed to cardboard using hot glue

- Routed for clean wiring

Spacing was important to avoid light leakage into neighboring squares, ensuring clean illumination.

7.4.3 Power Connections

Since 64 LEDs can consume up to 4A at full brightness:

- A 5V 5A SMPS was used

- Power was injected from two sides to reduce voltage drop

- A 1000 µF capacitor was added across VCC-GND

Initially, you accidentally used a 47 µF capacitor which caused:

- Low voltage

- Flickering

- Unstable LED behavior

Replacing it with the correct value solved the issue.

7.5 Integrating Arduino Mega 2560

7.5.1 Pin Assignments

- Rows: 39, 41, 43, 45, 47, 49, 51, 53

- Columns: 4, 5, 6, 7, 8, 9, 10, 11

- LED Data: Pin 13

Wires were kept long enough to reach Arduino but short enough to minimize noise.

7.5.2 Common Grounding

Initially, the LEDs showed:

- Garbage colors

- Random flicker

- Unpredictable patterns

This was because Arduino GND and SMPS GND were not connected.

Connecting all grounds together fixed the issue.

This highlights why grounding is critical in embedded systems.

7.6 Software Upload & Integration

7.6.1 Software Flow Testing

Integration of software with hardware involved:

- Testing row and column scanning

- Checking that lifting each piece shows correct changes in curState[][]

- Verifying LED mapping with serpentine index

- Ensuring correct pieceBoard initialization

You tested:

- Knights

- Rooks

- Bishops

- Queen

- Pawn forward and diagonal capture moves

7.6.2 Debugging Through Serial Monitor

Bugs were identified using serial logs:

- Wrong turn error

- Ghost lifting

- Misplaced capture

- Check & checkmate events

The system responded correctly to all these cases.

7.7 Testing and Validation

Testing was done in multiple stages.

7.7.1 Reed Switch Testing

Each of the 64 reed switches was tested individually by:

- Bringing a magnet close

- Monitoring column output

- Confirming detection on Arduino

Failures included:

- Switches that were too stiff

- Switches that broke

- Switches placed too far from surface

These were replaced or re-mounted.

7.7.2 Row-Column Testing

All 8 rows and 8 columns were tested for:

- Shorts

- Open circuits

- Incorrect diode orientation

- Noise

- Ghosting

Final matrix was stable after fixing several mistakes.

7.7.3 LED Testing

LED testing included:

- Full white mode

- Rainbow animation

- Single-pixel test

- Startup animation

- Win animation

Flicker issues were fixed by grounding and capacitor correction.

7.7.4 Move Logic Testing

You validated legal moves manually:

- Tried illegal rook moves

- Knight jumping

- Bishop diagonals

- Pawn two-step and capture

- Queen combinations

- King movements

Illegal moves were consistently blocked.

7.7.5 Simulation Testing (HTML + JavaScript)

Before porting to Arduino C, you tested logic by:

- Simulating board using Proteus (but reed simulation was difficult)

- Converting logic to HTML/JavaScript

- Using AI to debug move logic

- Porting final logic back to Arduino C

This innovative approach reduced logical bugs significantly.

7.8 Final Integration & Enclosure

After fixing all electrical and software issues:

- Reed matrix was fixed under LEDs

- LED matrix was fixed under chessboard print

- Glass sheet was placed on top

- Sides were sealed with cardboard frame

- Arduino was mounted at the back with USB access

- SMPS was mounted securely

- Wires were tied to avoid movement

Final board appeared professional, modern, and durable.

7.9 Challenges Faced During Implementation

You encountered several real-world problems:

1. Reed switches breaking easily

Glass reed switches cracked from heat or pressure.

2. Magnet alignment issues

Magnets were sometimes weak or at wrong height → required lowering board surface.

3. Wrong capacitor selection

47 µF instead of 1000 µF caused LED power instability.

4. Common ground not connected

Caused garbage LED output until fixed.

5. Serpentine arrangement confusion

LED indexing had to be corrected in code.

6. Physical soldering injuries

Hot glue and soldering iron caused hand cuts and burns.

These challenges make your implementation realistic and impressive.

7.10 Chapter Summary

This chapter explained the complete implementation process of the Smart Move Chess Board, including physical construction, wiring, testing, debugging, and final assembly. The system went through several iterations and corrections before reaching a stable prototype. The final product is a well-integrated embedded system combining sensors, LEDs, microcontroller logic, and a visually guided chess-playing experience.

CHAPTER 8

TESTING AND RESULTS

Testing is one of the most critical phases of any embedded system project. It ensures that the implemented hardware and software operate correctly under real-world conditions. The Smart Move Chess Board underwent rigorous testing at multiple levels—component-level, module-level, system-level, and gameplay-level.

This chapter systematically presents:

- Test procedures

- Test cases and expected results

- Actual results observed

- Challenges and fixes applied

- Performance evaluation

- Final assessment of system behavior

Testing validates that the system functions reliably, handles errors, and provides a consistent interactive chess-learning experience.

8.1 Objectives of Testing

The primary objectives were:

- Verify the correctness of reed switch matrix sensing

- Ensure LED feedback accuracy

- Validate chess rule enforcement

- Test response time and sensitivity

- Identify and eliminate hardware/software bugs

- Ensure robustness against user mistakes

- Confirm full-board functionality during gameplay

8.2 Testing Methodology

A systematic approach was used:

8.2.1 Unit Testing

Each component was tested individually:

- Reed switches

- Diodes

- Rows & columns

- LEDs

- Power stage

8.2.2 Integration Testing

Groups of components were tested together, such as:

- Reed matrix + Arduino

- Arduino + LED matrix

- Game logic + LED guidance

8.2.3 System Testing

Complete board testing included:

- Legal move validation

- Detection accuracy

- Error handling

- LED animations

- Full gameplay simulation

8.2.4 User Testing

Trial games played to evaluate usability and clarity of LED hints.

Fig 9 : Logic Testing

8.3 Test Case Summary Table

Below is a standard testing table expected in academic-level reports:

8.3.1 Hardware-Level Testing

Table 8.1 – Reed Switch Testing

| Test Case | Expected Output | Actual Output | Result |
| --- | --- | --- | --- |
| Bring magnet near reed | Column input goes LOW | Correct detection | PASS |
| Remove magnet | Column goes HIGH | Correct detection | PASS |
| Reed mounted too deep | No activation | Failed initially; fixed by height adjustment | FIXED |
| Broken reed | No response | Detected and replaced | PASS |

Table 8.2 – Row/Column Connectivity

| Condition | Expected | Actual | Result |
| --- | --- | --- | --- |
| Row HIGH, no magnet | All columns HIGH | Correct | PASS |
| Row LOW + magnet present | Column reads LOW | Correct | PASS |
| Incorrect diode polarity | Ghosting detected | Fixed after orientation correction | FIXED |
| Short circuit between rows | Multiple triggers | Fixed by rewiring | FIXED |

Table 8.3 – LED Functionality

| Test | Expected | Actual | Result |
| --- | --- | --- | --- |
| Startup animation | Cyan–teal wave | As expected | PASS |
| Legal move LED | Green on correct squares | Accurate | PASS |
| Capture LED | Red on enemy square | Accurate | PASS |
| Error blink | Red blinking | Visible clearly | PASS |
| Check indication | Yellow on king | Accurate | PASS |
| High brightness test | No flicker | Flicker before fixing capacitor | FIXED |

8.4 Software-Level Testing

8.4.1 State Machine Testing

Table 8.4 – State Handling

| Action | Expected State | Actual State | Result |
| --- | --- | --- | --- |
| Lift own piece | HOLDING | Correct | PASS |
| Lift opponent piece | WRONG_TURN | Correct | PASS |
| Place piece on illegal square | ERROR | Correct | PASS |
| Capture legally | CAPTURE_PENDING → IDLE | Correct | PASS |
| Checkmate scenario | GAME_OVER | Correct | PASS |

8.4.2 Legal Move Testing for Each Piece

Every piece was tested manually:

Pawn

- Forward 1-step: PASS

- Forward 2-step on start: PASS

- Diagonal capture: PASS

- Illegal backward movement: BLOCKED (PASS)

Rook

- Straight-line path: PASS

- Path blocked test: PASS

Knight

- Jump movement: PASS

- Illegal diagonal/straight: BLOCKED (PASS)

Bishop

- Diagonals only: PASS

Queen

- Combination of rook+bishop: PASS

King

- 1-step in any direction: PASS

- Illegal move into check: BLOCKED (PASS)

This confirms correctness of the move-generation algorithm.

8.5 System Integration Results

8.5.1 Movement Detection Accuracy

- All 64 squares detected correctly

- No ghosting after diode correction

- Response time < 40 ms (LEDs light instantly)

Accuracy Score: 100% stable detection

8.5.2 LED Guidance Accuracy

LEDs correctly displayed:

| Event | LED Indication | Result |
| --- | --- | --- |
| Legal moves | Green | PASS |
| Capture | Red | PASS |
| Wrong turn | Red origin highlight | PASS |
| Check | Yellow on king | PASS |
| Win | Blue/Teal wave animation | PASS |

8.5.3 Full Game Simulation

Game 1 – Sample Game Play

- Opening moves played correctly

- LED hints were accurate

- Captures handled flawlessly

- Check and checkmate functions worked

- Game ended with correct visual animation

Game 2 – Stress Test with Fast Movements

- Rapid lifts and placements tested

- System handled movement smoothly

- No skipped detection

- No unintended triggers

Game 3 – Illegal Move Stress Test

Tried intentionally illegal actions:

- Moving opponent’s piece

- Lifting two pieces at once

- Placing piece on wrong square

System blocked each illegal action successfully.

8.6 Bugs Found and Fixes Applied

During testing, several major and minor bugs were discovered.

8.6.1 Bug 1 – Dead LEDs or Wrong Colors

Cause:
Poor grounding and insufficient capacitor.

Fix:

- Connected Arduino and LED ground together

- Added 1000 µF capacitor

- Reduced brightness to 140

8.6.2 Bug 2 – Ghosting in Matrix

Cause:
Diodes missing in initial wiring.

Fix:
Added 1N4148 diode for each switch.

8.6.3 Bug 3 – Wrong LED Mapping

Cause:
Serpentine indexing not implemented.

Fix:
Added mapping function ledIndex().

8.6.4 Bug 4 – Illegal move not blocked

Cause:
Incorrect board simulation in legality check.

Fix:
Corrected king safety simulation and restore mechanism.

8.6.5 Bug 5 – Capture Logic Issues

Cause:
Capture path not validated in holding stage.

Fix:
Added CAPTURE_PENDING state.

8.6.6 Bug 6 – Power Fluctuation

Cause:
Using a 47 µF capacitor instead of 1000 µF.

Fix:
Correct capacitor installed.

8.7 Performance Evaluation

| Parameter | Expected | Achieved | Remarks |
| --- | --- | --- | --- |
| Detection Time | < 50 ms | ~40 ms | Excellent |
| LED Response | Instant | Yes | Smooth |
| Illegal Move Blocking | 100% | Yes | Very robust |
| Check Detection | Accurate | Yes | Matches rule engine |
| Power Stability | Stable | Stable after fixes | Good |
| User Interaction | Beginner-friendly | Yes | Visually clear |

8.8 Limitations Identified During Testing

Although the system performs well, certain limitations exist:

- Reed switches are fragile

- Magnets must align perfectly

- Castling, en-passant, auto-promotion not implemented

- LED and reed layers increase thickness

- Multiple fast illegal actions can temporarily confuse piece display

- Board is limited to chess only (future enhancement planned)

8.9 Chapter Summary

This chapter provided a complete overview of the testing strategy, results, observations, and error corrections. The Smart Move Chess Board successfully passed all major hardware, software, and gameplay tests. Real-world testing confirmed that the system operates reliably, accurately detects movements, enforces rules, and provides education-oriented visual guidance.

The next chapter will describe applications, future scope, and potential enhancements.

CHAPTER 9

APPLICATIONS & FUTURE SCOPE

The Smart Move Chess Board is not just a hobby project—it is a functional educational tool with real-world applications in learning, gaming, embedded systems, AI integration, and IoT. This chapter highlights the practical uses of the system today as well as its potential to evolve into a more advanced and commercially viable product in the future.

9.1 Applications of Smart Move Chess Board

9.1.1 Educational Tool for Beginners

The board helps users learn chess by:

- Showing legal moves

- Highlighting illegal attempts

- Indicating check and checkmate

- Providing visual guidance through LEDs

This makes it suitable for:

- Schools

- Coaching academies

- Beginner chess clubs

- Self-learning environments

9.1.2 Training Tool for Intermediate Players

The board reinforces correct chess concepts:

- Piece behavior

- Tactical awareness

- Move consequences

- Avoiding mistakes

Players can physically interact with the board while receiving digital assistance.

9.1.3 Embedded Systems Learning Platform

This project is an excellent educational example for embedded systems because it includes:

- Sensor matrix scanning

- Multiplexing

- LED driving

- State-machine programming

- Real-time processing

- Debugging techniques

Electronics students can learn practical skills through this system.

9.1.4 Digital Gaming Interface

With further enhancement, the board can:

- Interface with chess engines

- Connect to mobile apps

- Log moves for analysis

- Play against AI physically

This brings digital advantages to a physical board.

9.1.5 Advance Human-Computer Interaction Prototype

This system demonstrates an early foundation of physical–digital hybrid systems used in:

- Smart toys

- Augmented reality interfaces

- Interactive boards

- Game training systems

- Smart learning kits

The project is scalable and adaptable for multiple smart-board applications.

9.1.6 Rehabilitation / Cognitive Training

Chess is used for:

- Cognitive improvement

- Memory development

- Rehabilitation therapy

This smart board provides clear visual assistance, making it suitable for:

- Elderly users

- People recovering from cognitive impairment

- Children with learning difficulties

9.2 Future Scope of Smart Move Chess Board

The current version is already functional and reliable, but there are many exciting possibilities for expansion.

9.2.1 Bluetooth / Wi-Fi Integration

Adding ESP32 or HC-05 Bluetooth modules would allow:

- Mobile app connectivity

- Remote coaching assistance

- Online play

- Move recording

- Cloud-based analysis

This transforms the board from a standalone system into an IoT-enabled smart device.

9.2.2 Integration with Stockfish Chess Engine

A powerful step forward is to enable the board to:

- Play against Stockfish AI

- Suggest best moves

- Analyze positions

- Train users using puzzles

Stockfish can run externally on:

- Laptop

- Raspberry Pi

- Mobile app

The board would act as an intuitive physical interface.

9.2.3 Game Recording & PGN Export

The board can log moves automatically and export them in:

- PGN (Portable Game Notation)

- JSON

- Text format

Useful for:

- Tournament preparation

- Player improvement

- Sharing games online

9.2.4 Replacing Reed Switches with Hall Sensors

Hall sensors provide:

- Higher durability

- Better consistency

- Identification of piece type (if using different magnet strengths)

This eliminates reed fragility and improves long-term reliability.

9.2.5 Battery Power Integration

A Li-ion battery with charging circuitry would make the board:

- Portable

- Independent of wired power

- Safer for transport

This helps in competitions and outdoor learning.

9.2.6 Adding a Touch Screen Interface

A small touchscreen can add:

- Game mode selection

- Difficulty levels

- Timer/clock

- Visual board on screen

- Rule explanations

This creates a hybrid smart board + digital UI device.

9.2.7 Making the Board Multi-Game Compatible

With LEDs and sensors, the board can support:

- Ludo (LED-guided movement)

- Sudoku (highlighting patterns)

- Checkers

- Snake path games

- Memory games

This drastically expands the product’s appeal.

9.2.8 Advanced Sensing & Automation

In future:

- Using RFID or computer vision can identify piece type automatically

- The board can show opponent moves physically

- Servo motors or electromagnets can move pieces (like Square Off boards)

While ambitious, these are realistic future improvements.

9.2.9 Integration with IoT and Cloud Systems

Connecting the board to IoT enables:

- Global multiplayer

- Cloud storage of games

- Remote coaching

- Online tournaments

This moves the board into the domain of professional smart devices.

9.3 Summary of Applications & Scope

The Smart Move Chess Board has diverse current applications and an extensive roadmap for future enhancement. It bridges physical gameplay with digital intelligence, making it suitable for learners, hobbyists, educators, and embedded-system developers. With additional connectivity, AI integration, and multi-game support, the board can evolve into a commercial-grade smart learning device.

CHAPTER 10

CONCLUSION

The Smart Move Chess Board successfully demonstrates the integration of hardware sensing, embedded logic, and interactive visual feedback to create an intelligent, beginner-friendly chess-playing system. The project is a strong example of applying engineering principles—both electronic and computational—to solve real-world educational challenges.

10.1 Summary of Achievements

The project accomplished the following key objectives:

✔ Accurate detection of chess piece movement using 64 reed switches

Each square reliably detects when a piece is lifted or placed.

✔ Real-time LED indication using WS2812B RGB LEDs

Legal moves, captures, errors, and critical game states are visually represented.

✔ Full chess rule enforcement

Illegal moves are blocked, wrong-turn actions are prevented, and check/checkmate are detected.

✔ Stable and optimized embedded system

The Arduino Mega processes sensor input and LED output efficiently, maintaining smooth gameplay.

✔ Clean, user-friendly physical board

Layers of reed switches and LEDs are enclosed neatly under a glass top, providing both functionality and aesthetics.

✔ Logical debugging and simulation

Game logic was validated using HTML/JavaScript before being ported to Arduino C, ensuring robustness.

10.2 Key Learnings

The development of this project provided experience in:

- Sensor matrix design

- Embedded system programming

- Real-time scanning and data processing

- LED driver management

- Complex move generator implementation

- Hardware debugging and soldering

- Power distribution and common grounding

- User-experience driven engineering

This combination of skills is highly valuable for future IoT, robotics, and embedded-system projects.

10.3 Limitations

Despite its success, the system has some limitations:

- Reed switches are fragile

- Magnet alignment must be precise

- Advanced rules like castling and en-passant are not yet implemented

- Piece type identification requires manual initialization

- Board thickness increases due to sensors and LEDs

These limitations open opportunities for future improvement.

10.4 Conclusion Statement

The Smart Move Chess Board is a well-designed and carefully implemented embedded system that enhances the physical game of chess through smart sensing and interactive LED guidance. It successfully bridges the gap between traditional gameplay and modern learning tools. The project demonstrates innovation, technical depth, practical engineering skills, and the potential for expansion into advanced IoT-based and AI-enhanced systems.

This project proves that low-cost components, when combined with smart engineering, can create powerful educational technologies.

10.5 Future Work Summary (Combined Statement)

The next steps for this project include:

- Adding Bluetooth/Wi-Fi connectivity

- Integrating Stockfish AI

- Recording and analyzing player moves

- Upgrading to Hall sensors

- Adding touchscreen modes

- Expanding the board to support multiple games

- Cloud-enabled IoT functionality

These enhancements would elevate the system from a prototype into a full-featured smart gaming platform.

REFERENCES

- Arduino Documentation, “Arduino Mega 2560 Technical Specifications,” Available at: https://docs.arduino.cc

- WS2812B LED Datasheet, WorldSemi, 2020.

- Reed Switch Technical Guide, “Magnetic Sensor Basics,” Meder Electronics / Standex-Meder, 2019.

- Neodymium Magnet Specifications, K&J Magnetics, Available at: https://www.kjmagnetics.com

- Atmel ATmega2560 Datasheet, Microchip Technology Inc., 2023.

- Paul Horowitz and Winfield Hill, The Art of Electronics, 3rd Edition, Cambridge University Press, 2015.

- Richard H. Barnett, Larry O’Cull, and Sarah Cox, Embedded Systems: Real-Time Interfacing to the 8051/Arduino, Thomson, 2018.

- Adafruit Industries, “NeoPixel LED Best Practices & Usage Guide,” Available at: https://learn.adafruit.com

- Wikipedia Contributors, “Chessboard,” Wikipedia, The Free Encyclopedia, Accessed 2025.

- Texas Instruments, “SMPS Design Basics: Power Supply Fundamentals,” TI Application Notes, 2021.

- TutorialsPoint, “Matrix Keyboard/Matrix Scanning Basics,” Available at: https://www.tutorialspoint.com

- GreatScott!, “How to Use WS2812B LEDs,” Video Tutorial, YouTube, 2019.